Technical Report:
Dual-Sensor HSI (NIR) and FTIR (MIR) Fallback Classification System

1. System Architecture Overview

The proposed system implements a dual-stage sensor-fusion fallback architecture designed for the classification of post-consumer plastic waste. The primary sensor stage consists of a high-speed Near-Infrared (NIR) Hyperspectral Imaging (HSI) camera. When the NIR sensor scan yields an inconclusive prediction—due to low reflectance signal (e.g. black plastics containing carbon black colorants) or non-plastic material categories (e.g. paper labels or food waste)—the pipeline automatically triggers the high-precision Fourier Transform Infrared (FTIR) Mid-Infrared (MIR) spectrometer. The secondary scan operates as a backup to check the chemical fingerprint of the underlying polymer and save the plastic item from misclassification.

2. Dataset Specifications

A. Primary NIR HSI Dataset

• Source: Specim FX10 Hyperspectral camera ('ADVANCED_PLASTIC_FUSE' dataset).

• Spectral Configuration: 232 spectral bands spanning the visible to Near-Infrared spectrum (400 - 1000 nm).

• Spatial Configuration: 1,220 subimages structured as 3D HSI data cubes of size 64 x 64 pixels x 232 bands.

• Splits: Stratified split containing 731 samples for training, 246 for validation, and 243 for testing.

• Target Classes: High-Density Polyethylene (HDPE, 178 samples), Polypropylene (PP, 335 samples), Low-Density Polyethylene (LDPE, 178 samples), Polyethylene Terephthalate (PET, 226 samples), Polystyrene (PS, 43 samples), Organic Contamination (ORGANIC, 145 samples), and Other Waste (OTHER, 115 samples).

B. Backup MIR FTIR Dataset

• Source: Mid-Infrared Fourier Transform Infrared (FTIR) spectrometer ('FTIR-PLASTIC' dataset).

• Spectral Configuration: 3,736 spectral wavenumbers spanning the Mid-Infrared region (399.19 - 4000.60 cm⁻¹) measured at a high resolution of 4 cm⁻¹.

• Split: Stratified split containing 1,853 samples for training, 398 for validation, and 398 for testing.

• Target Classes: Balanced dataset containing PET, HDPE, LDPE, PP, PS, and PVC.

3. Preprocessing Pipelines

• NIR HSI Preprocessing: Spatial dimensions are reduced by calculating the spatial mean spectrum of the 64 x 64 subimage. This reduces each data cube into a 1D vector of length 232, which is then normalized using Z-score column scaling (StandardScaler).

• MIR FTIR Preprocessing: Raw 3,736-point spectra are smoothed using a Savitzky-Golay filter (window length = 15, polynomial order = 2, derivative order = 0) to remove high-frequency noise. Standard Normal Variate (SNV) normalization is applied individually to each spectrum to correct for physical path length and light-scattering variations. Features are then Z-score column scaled.

4. Machine Learning Models & Performance

• NIR Classifier: Support Vector Machine (SVM) utilizing a Radial Basis Function (RBF) kernel (regularization parameter C = 10.0, gamma = 'scale'). It achieved a test accuracy of 97.12% and a weighted F1-score of 97.11%.

• MIR Classifier: Random Forest (RF) classifier consisting of 200 estimators (decision trees) with a max depth of 15. The model achieved a test accuracy and weighted F1-score of 100.00%.

5. Physical Interpretation & Wavenumber Bands

Molecular bonds absorb infrared radiation at highly specific frequencies, which provides the physical basis for polymer identification. The following key absorption regions are utilized by the MIR classifier:

| Wavenumber Range (cm⁻¹) | Vibration / Bond Type | Polymer Identified |
| --- | --- | --- |
| 1700 - 1750 | C=O (Carbonyl) stretching vibration | PET (Ester linkage) |
| 2800 - 3000 | C-H (CH2 and CH3) stretching | HDPE, LDPE, PP (Polyolefins) |
| 1375 | C-H bending of methyl group | PP (Separates PP from PE) |
| 1450 - 1600 & 700 - 760 | Aromatic ring C=C / C-H bends | PS (Polystyrene) |
| 600 - 700 | C-Cl stretching | PVC (Chlorine bond) |

Black plastics containing carbon black pigment absorb almost all Near-Infrared (NIR) photons, dropping average reflectance to less than 8%. Under these conditions, the NIR model fails. However, in the Mid-Infrared (MIR) range, the light's wavelength (2.5 - 25 µm) is much larger than the carbon black pigment particles, leading to minimal scattering and absorption. Thus, the MIR spectrometer beam successfully interacts with the polymer chains, resolving clean finger-print peaks (like C-H stretching for PP or C-Cl for PVC) to classify the black plastic sample with 100% precision.